# M5Stack Tab5 and LVGL

by Bryan A. “CrazyUncleBurton” Thompson

Last Updated 2/21/2026

## The Platform IO Project

The Platform IO project is located here:

https://github.com/CrazyUncleBurton/M5Stack-Tab5-LVGL

See the document “VS Code with Source Control.pdf” located in the same directory as this document.

## Testing

- Connect a USB cable from the computer that VS Code is running on to the M5Stack microcontroller.

- If the microcontroller is not on, press the white button on the side once to turn it on.

- In VS Code, click the PlatformIO Upload icon  in the bottom toolbar.  The hover text will be PlatformIO:  Upload.

- The project will compile.  When it’s ready to upload, we see lines like “Connecting…” and “Writing…”.  Wait for the line that says SUCCESS in green text before proceeding.

- When finished, the microcontroller will reboot.

- When the microcontroller reboots, watch the display.  You should see a touch-controlled slider, a touch-controlled arc slider, and a touch button.  Pressing the button increases the other sliders by 10.  Sliding one slider makes the other move.

## Reference

Tab5 Microcontroller Info:  https://docs.m5stack.com/en/core/Tab5

LVGL Docs: https://docs.lvgl.io/master